## E S T I M A T E

#### DIVISION

SUB-DIVISION FUND HEAD MAJOR HEAD MINOR HEAD SERVICE HEAD

DEPARTMENTAL HEAD

Estimate framed in the Office of

Satara for the probable expenses that will be incurred for the work of Construction of Godown Builidng For Chandaneshwar Agro Producer Company Ltd. In survey No 502 At:—Banawadi Tal:—Koregaon Dist:— Satara.

Owner Name— Mr.Ramchandra Kisan Nalage.

As.	69,41,000.00

Estimate Prepared by : Estimate Checked by Technical sanction Dated

Administrative Approval : Dated :

GENERAL DESCRIPTION

—	As per separate sheet attached

Agro Producer Company Ltd. In survey No 502 At:-Banawadi Tal:-Koregaon Dist:-  Satara.  owner Name- t•tr. chandra Kisan Nalage.

RECAPITULATION-SHEET

##### Sub-Estimate No & Name	Amount

1 Godown

Add G.S.T. on Residencial Buildings

Add

| (c) | *°. | 69,41,016.00 |
| --- | --- | --- |
|  | Rs. | 333.00 |
|  | As. | 69, 41,349.00 |
| Say | Rs. | 69, 41, 000.00 |

@ 12.00% On (B)

Rs.

##### Rs.

61,97,336.00

##### 7,43,680.00

Agro Producer Company Ltd. In survey No 502 At:-Banawadi  Tal:-Koregaon  Dist:-  Satara. owner Name- t•tr. chandra Kisan Nalage.

A B S T R A C T

###### Quantity

ITEM NO : 1

###### Description

Aate	Unit

Amount

168.150

Cubic metre

Excavation for foundation in earth, soils of all types, sand, gravel, soft murum including shoring and strutting as necessary and disposing off excavated stuff as directed, excetra complete. (By Manual Means)

247.05 One

Cubic Metre

41541.46

SSR 2021—22 HP	12.02*64

Deduct 2.0% for Labour aminities & Cess

(—2.0	100 * 225.00)

Add 10.00% For Hilly*Inacce. Area

Add	Back	2.0%	for Labour aminities \ Cess				(2.0	100 * 225.00)

ITEM NO : 2

225.00

—4.50

2 2 0 . 5 0

2 2 . 0 5

4.50

247.05

36.680

Cubic Metre

Excavation for foundation in hard murum including shoring and  strutting  as necessary and disposing off excavated stuff as directed, excetra complete.

SSR 2021—22 HP 12.04*64	222.00

Deduct 2.0% for Labour aminities \ Cess

243.75 One

Cubic Metre

8940.75

(—2.0	100 * 222.00)

Add 10.00% For Hilly*Inacce. Area

Add	Back	2.0%	for Labour aminities \ Cess				(2.0	100 * 222.00)

ITEM NO : 3

—  4 . 4 4

217.56

21.76

4.44

243.76

45.990

Cubic metre

Providing soling using 80 milimetre size trap metal in 15 centimetre layer including filling voids with crushed sand grit, ramming, watering, excetra complete.

Rate as per R. A. No.— 1

2068.40 One

Cubic Metre

95125.72

22.590

Cubic Metre

ITEM NO : 4

Providing and laying Cast in situ*Beady Mix, cement concrete		in	M—10		of		trap*		granite*		quartzite* gneiss	metal		for foundation and bedding including bathing		out		water,		formwork,		laying*		pumping, compacting and curing, excetra complete, with fully automatic		micro processor based Programmable Logic Controller			without		Supervisory		Control		and Data Aquisition			enabled		reversible		Drum	Type			mixer* concrete			Batch		mix	plant	(Pan	mixer)		excetra complete.		With	fine		aggregate (Natural Sand Crushed sand VSI Grade finely washed, excetra) (using NATURAL SAND).

Rate as per R. A. No.	2

6570.95 One	148437.76

Cubic Metre

###### 294045.69

36.680

Cubic Metre

ITEM NO : 5

Filling in plinth and floors with approved excavated materials in 15 centimetre to 20 centimetre layers including watering and compacting, excetra complete.

124.05 One

Cubic Metre

4550.15

SSR 2021—22 I P 21.36 117

Deduct 2.0% for Labour aminities & Cess

(—2.0	100 * 113.00)

Add 10.00% For Hilly*Inacce. Area

Add	Back	2.0%	for Labour aminities \ Cess				(2.0	100 * 113.00)

ITEM NO : 6

113.00

—2.26

110.74

11.07

2.26

124.07

175.580

Cubic Metre

38.380

Cubic Metre

Filling in plinth and floors with contractors material brought from outside and approved by Engineer In—Charge, in 15 centimetre to 20 centimetre layers including watering and compaction, excetra complete.

Rate as per R. A. No.— 8

ITEM NO : 7

Providing	uncoursed	rubble	masonry	of	trap granite	quartzite		gneiss	stones		in cement mortar 1:6 in foundations and plinth of inner walls

in plinth of external walls including bathing out water, striking joints on unexposed faces and watering, excetra complete. (using NATURAL SAND).

Rate as per R. A. No.— 20

823.95 One	144669.14

Cubic Metre

6019.60 One	231032.25

Cubic Metre

6.030

Cubic Metre

ITEM NO : 8

Providing second class burnt brick masonry with conventional Indian Standard type  bricks  in cement mortar 1:6 in foundations and plinth of inner walls in plinth of external walls including bathing out water manually, striking joints, on unexposed faces, racking out joints on exposed faces and watering, excetra complete. (using NATURAL SAND).

Rate as per R. A. No.—	9

ITEM NO : 9

9103.00 One

Cubic Metre

54891.09

49.550

Cubic metre

Providing second class Burnt Brick masonry with conventional Indian Standard type  bricks  in cement mortar 1:6 superstructure including striking joints, racking out joints, watering and scaffolding, excetra complete. (For Ground Floor) (using NATURAL SAND).

Rate as per R. A. No.—	10

9352.25 One	463403.99

Cubic Metre

|  | ITEM NO : 10 |  |  |
| --- | --- | --- | --- |
| 52.550 | Providing and laying cement concrete blocks of M—15 | 8434.95 | One 443256.62 |
| Cubic | grade flooring laid in cement mortar 1:3 as per |  | Cubic |
| metre | drawing and technical specifications, excetra |  | Metre |

complete. (using NATURAL SAND). Rate as per R. A. No.—	11

###### 1635848.93

1140.860

Square Metre

ITEM NO : 11

Providing and fixing corrugated galvanised iron sheets of 0.63 milimetre thick (24 B.W.G) for roofing without wind tiles including fastening with galvanised iron screw and bolts and lead bitumen washers  as  per drawing, excetra complete. (Weight

5.5 killogram per square metre) (For Ground Floor) SSR 2021—22 HP 38.04 171	816.00

Deduct 2.0% for Labour aminities \ Cess

(—2.0 100 * 816.00) —16.32

895.95 One	1022153.52

Square Metre

Add 10.00% For Hilly*Inacce. Area

Add	Back	2.0%	for Labour aminities \ Cess				(2.0	100 * 816.00)

ITEM NO : 12

799.68

79.97

16.32

895.97

11.731

Metric

Ton

38.880

Square Metre

Providing structural steel work in trusses, other similar trussed purlins and members with all bracing, gusset plates excetra as per detailed desings and drawings or as directed including cuting, fabricating, hoisting, erecting, fixing in position, making rivetted  bolted  welded connection and one coat of anticorrosive paint and over it two coats of oil painting of approved quality and shade, excetra complete. (For Ground Floors

Rate as per R. A. No.— 12

ITEM NO : 13

Providing			and	fixing		rolling		shutter fabricated from steel laths of minimum thickness 0.9 milimetre with			lock				plate		of		3.15	milimetre	thickness reinforced			with		35			milimetre		X Z5 milimetre X 5 milimetre			angle		section fitted with sliding bolts and handles for both sides, deep mild steel channel section		of			depth		and		thickness not less than 65 milimetre and 3.15 milimetre respectively with hold fast	arrangements,		mild	steel		bracket plate 300 milimetre			X 300 milimetre X 3.15 milimetre minimum size and shape with square bar, suspension shaft of minimum		32			milimetre diametre, hood cover of mild steel		sheet			not less than 0.9 milimetre thickness and of any size at top and safety devices including mechanical gear operation arrangement consisting of worm		gear wheels and worms of high grade cast iron or	mild		steel and one coat of red lead primer, excetra complete. (Indian Standard : 6248—1979) without mechanical gear (For Ground Floor)

116598.05 One	1367811.72

Metric Ton

4732.40 One	183995.71

Square Metre

SSR 2021—22 HP	39.24*181

Deduct 2.0% for Labour aminities & Cess

4310.00

(—2.0	100 * 4310.00)	—86.20

4223.80

Add 10.00% For Hilly*Inacce. Area

Add	Back	2.0%	for Labour aminities \ Cess				(2.0	100 * 4310.00)

ITEM NO : 14

422.38

86.20

4732.38

395.860

Square Metre

Providing internal cement plaster 20 milimetre thick in single coat in cement mortar 1:5 without neeru finish, to concrete, brick surface, in all positions including scaffolding and curing, excetra complete. (For Ground Floor) (using NATURAL SAND).

Rate as per R. A. No.—	13

406.45 One	160897.30

Square Metre

ITEM NO : 15

255.620	Providing		sand	faced plaster externally in cement Square	mortar	using	approved	screened	sand	in	all

674.40 One	172390.13

Square

###### 4543097.31

Metre

525.460

Square Metre

positions including base coat of 15 milimetre thick in cement mortar 1:4 using waterproofing compound at one killogramme per cement bag curing the same for not less than 2 days and keeping the surface of the base coat rough to receive the sand faced treatment of 6 to 8 milimetre thick in cement mortar 1:4 finishing the surface by taking out grains and curing for fourteen days, scaffolding, excetra complete. (For Ground Floor) (using NATURAL SAND).

Rate as per R. A. No.—	14

ITEM NO : 16

Providing and laying vitrified matt finish tiles having size 590 milimetre to 605 milimetre X to 605 milimetre of 8 milimetre to 10 milimetre thickness and confirming to Indian Standard 15622—2006 (Group BIa) of approved make, shade and pattern for dado and skirting in required position laid on a bed of 1:4 cement mortar including neat cement float, filling joints, curing and cleaning, excetra complete. — Skirting (For Ground Floor) (using NATURAL SAND).

Rate as per R. A. No.—	15

ITEM NO : 17

Metre

2214.75 One	1163762.54

Square Metre

21.600

Square Metre

Providing and fixing mild steel grill work for windows, ventilator, excetra 20 kilogram per square metre as per drawings including fixtures, necessary welding and painting with one  coats  of anticorrosive paint and two coats of oil painting, excetra complete. (For Ground Floor)

Rate as per R. A. No.— 16

2032.55 One

Square Metre

43903.08

###### 5750762.93

|  | ITEM NO : 18 |  |  |
| --- | --- | --- | --- |
| 21.600 | Providing and fixing in position (as per Indian | 6396.95 | One 138174.12 |
| Square | Standard : 1868*1982) aluminium sliding window of |  | Square |
| Metre | three tracks with rectangular pipe having overall |  | Metre |
|  | dimentsions 95 milimetre X 38 milimetre X 0.90 |  |  |
|  | milimetre at weight 0.637 kilogram per running |  |  |

metre and window frame bottom track section 92 milimetre X 31.75 milimetre X 1.30 milimetre at weight 1.070 kilogram per running metre. Top and side track section 92 milimetre X 31.75 milimetre X

1.30 milimetre at weight 0.933 kilogram per running metre. The shutter should be of bearing bottom 40 milimetre X 18 milimetre X 1.25 milimetre  at weight

0.417 kilogram per running metre. Interlocking secion 40 milimetre X 18 milimetre X 1.10 milimetre at weight 0.469 kilogram per running metre and handle section 40 milimetre X 18 milimetre X 1.25 milimetre at weight 0.417 kilogram per running metre and top section 40 milimetre X 18 milimetre X

1.25 milimetre at weight 0.417 kilogram per running metre as per detailed drawing and as directed by Engineer in—charge with all necessary aluminium sections, fixtures and fastenings such as roller bearing in nylon casting and self locking catch fitted in vertical section of shutter including 5 milimetre thick plain glass and aluminium mosquito net shutter with stainless steel jali with all required screws and nuts excetra complete. — With colour anodising with box. (For Ground Floor)

SSR 2021—22 HP 39.41 184	5826.00

Deduct 2.0% for Labour aminities \ Cess

(—2.0    100 * 5826.00)   —116.52

5709.48

Add 10.00% For Hilly*Inacce. Area

Add	Back	2.0%	for Labour aminities \ Cess				(2.0	100 * 5826.00)

ITEM NO : 19

570.95

116.52

6396.95

12.300

Square Metre

Providing and fixing solid core flush door shutter in single leaf 32 milimetre thick decorative type of exterior grade as per detailed drawings, approved face veneers 3 milimetre thick on both faces or as directed, all necessary beeds, moulding and tipping, wrought iron hold fasts, chromium plated fixtures and fastenings, with brass mortise lock, chromium plated handles on both sides and finishing with french polish, excetra complete. (For Ground Floor)

3100.75 One

Square Metre

38139.23

SSR 2021—22 HP	39.09 179

Deduct 2.0% for Labour aminities & Cess

2824.00

(—2.0	100 * 2824.00)	—56.48

2767.52

Add 10.00% For Hilly*Inacce. Area

Add	Back	2.0%	for Labour aminities \ Cess				(2.0	100 * 2824.00)

276.75

56.48

3100.75

###### 5927076.28

19.800

Running Metre

ITEM NO : 20

Providing and fixing cold rolled steel hollow door frame 105 milimetre X 60 milimetre for any sizes including hold fast, hinges, tie members with red oxide and gray paint, excetra complete as per Indian Standards Institution specifications and code of practice prescribed including anticorosive treatment, excetra complete as directed by the Engineer. (For Ground Floor)

1997.25 One

Running Metre

3 9 5 45 . 5 5

SSR 2021—22 I P 39.49*186

Deduct 2.0% for Labour aminities & Cess

1819.00

(—2.0	100 * 1819.00)	—36.38

1782.62

Add 10.00% For Hilly*Inacce. Area

Add	Back	2.0%	for Labour aminities \ Cess				(2.0	100 * 1819.00)

ITEM NO : 21

178.26

36.38

1997.26

395.860

Square metre

Providing and applying washable oil bound distemper of approved colour and shade to old and new surfaces in two coats including scaffolding, preparing the surfaces (excluding the primer coat), excetra complete. (For Ground Floor)

47.20 One

Square Metre

18684.59

SSR 2021—22 HP	36.11 167

Deduct 2.0% for Labour aminities & Cess

(—2.0	100 * 43.00)

Add 10.00% For Hilly*Inacce. Area

Add	Back	2.0%	for Labour aminities \ Cess				(2.0	100 * 43.00)

ITEM NO : 22

43.00

—0.86

0.86

47.21

255.620

Square Metre

Providing and applying two coats of exterior weather shield paint of approved manufacturer and of approved colour to the plastered surfaces including cleaning, preparing the plaster surfaces, applying primer coat, scaffolding, if necessary and watering the surface for two days, excetra complete. (For Ground Floor)

261.30 One

Square Metre

66793.51

SSR 2021—22 HP	35.26 165

Deduct 2.0% for Labour aminities & Cess

(—2.0	100 * 238.00)

Add 10.00% For Hilly*Inacce. Area

Add	Back	2.0%	for Labour aminities \ Cess				(2.0	100 * 238.00)

238.00

—4.76

4.76

261.32

###### 6052099.93

0.590

Cubic Metre

ITEM NO : 23

Providing		and		laying	cast	in	situ		Ready Mix cement		concrete M—20 of trap* granite	quartzite* gneiss		metal		for reinforced cement concrete slabs and	landings		as per detailed designs and drawings including				steel		centering,	formwork,		cover blocks,laying				pumping, compaction, finishing the formed			surfaces		with		cement		mortar		l:3	of sufficient		minimum		thickness to give a smooth and even		surface		or roughening the surface if special finish	is		to		be	provided	and curing, excetra complete. (Excluding reinforcement and structural steel). with fully automatic micro processor based Programmable Logic Controller without Supervisory Control and Data Aquisition enabled reversible Drum Type mixer* concrete Batch mix plant (Pan mixer) excetra complete. With fine aggregate (Natural Sand Crushed sand VSI Grade finely washed etc) (For

Ground Floor) (using NATURAL SAND). Rate as per R. A. No.—	21

15762.60 One

Cubic Metre

9299.93

|  | ITEM NO : 24 |  |  |
| --- | --- | --- | --- |
| 3.000 | Providing and fixing Orissa type colour glazed | 10133.45 | One 30400.35 |
| Number | earthenware 625 milimetre X 450 milimetre water |  | Number |
|  | closet pan including trap, cast iron soil and vent |  |  |
|  | pipe upto the out side face of wall including 100 |  |  |

milimetre diameter cast iron plug bend, 15 milimetre thick 10 litre polyvinyl chloride flushing cistern with all necessary pipe connection, excetra complete. (For Ground Floor)

SSR 2021—22 HP 42.35*202	9229.00

Deduct 2.0% for Labour aminities & Cess

(—2.0    100 * 9229.00)   —184.58

9044.42

Add 10.00% For Hilly*Inacce. Area

Add	Back	2.0%	for Labour aminities \ Cess				(2.0	100 * 9229.00)

ITEM NO : 25

904.44

184.58

10133.44

3.000	Providing		and fixing white glazed earthenware wash Number	hand	basin		of		55 centimetre X 40 centimetre size including	cold	water	piller	tap	cold and hot

water piller tap brackts, rubber plugs and brass chain, stop tap and necessary pipe connections including polyvinyl chloride waste pipe and trap upto the outside face of the wall.Making good the damaged surface,testing excetra complete. (For Ground Floor)

5839.15 One

Number

17517.45

SSR 2021—22 I P 41.63*198

Deduct 2.0% for Labour aminities & Cess

5318.00

(—2.0	100 * 5318.00)	—106.36

5211.64

Add 10.00% For Hilly*Inacce. Area

Add	Back	2.0%	for Labour aminities \ Cess				(2.0	100 * 5318.00)

521.16

106.36

5839.16

6109317.66

45.000

Running Metre

ITEM NO : 26

Providing, laying and fixing polyvinyl chloride pipe of 15 milimetre diameter with fittings, such as bends, tees, reducers, clamps, excetra including necessary excavation, trench filling, excetra complete.Including removing existing pipe line if necessary and conveying and stacking the same in PWD chowky or as directed excetra complete.

84.55 One

Running Metre

3804.75

SSR 2021—22 I P 42.21 201

Deduct 2.0% for Labour aminities & Cess

(—2.0	100 * 77.00)

Add 10.00% For Hilly*Inacce. Area

Add	Back	2.0%	for Labour aminities \ Cess				(2.0	100 * 77.00)

ITEM NO : 27

77.00

75.46

7.55

84.55

23.000

Running Metre

Providing, laying and fixing polyvinyl chloride pipe of 110 milimetre diameter with fittings such as bends, tees, reducers, clamps, excetra including necessary excavation, trench filling, excetra complete.Including removing existing pipe line if necessary and conveying and stacking thesame in PWD chowky or as directed excetra complete.

587.45 One

Running Metre

SSR 2021—22 HP	42.28 202

Deduct 2.0% for Labour aminities & Cess

535.00

(—2.0	100 * 5Z5.00)	—10.70

524.30

Add 10.00% For Hilly*Inacce. Area

Add	Back	2.0%	for Labour aminities \ Cess				(2.0	100 * 535.00)

ITEM NO : 28

10.70

587.43

1500.000	Provoding		and		fixing	high		density		polyethylen Litre	container	of approved make one piece moulded water tank made out of high density polyethylen and built corrugated		inclusive	of delivery upto destination housting,		fixing		of	accessaries	such		as inlet, outlet,		overflow		pipe			inclusive	of		all	Taxes.

Capacity	between	200	litres to 1000 litres. (For Ground Floor)

14.25 One

Litre

21375.00

SSR 2021—22 HP	42.53*204

Deduct 2.0% for Labour aminities & Cess

(—2.0	100 * 13.00)

Add 10.00% For Hilly*Inacce. Area

Add	Back	2.0%	for Labour aminities \ Cess				(2.0	100 * 13.00)

ITEM NO : 29

13.00

—0.26

12.74

1.27

0.26

14.27

1.000	Providing	and	fixing	hume	pipe	septic tank 900 Number	milimetre diameter with vent pipe and cap including

excavation and laying.

Rate as per R. A. No.—	22

17540.20 One

Number

17540.20

###### 6165548.96

ITEM NO : 30

2.000	Providing and constructing brick masonry inspection Number	chamber	90		centimetre		X	45	centimetre	X	90 centimetre		including		1:4:8	cement		concrete foundation,	1:2:4	cement		concrete channels, half

round glazed stoneware pipes, brick masonary, plastering from inside and cast iron lid cover 75 kilogram with frame fixed in cement  concrete, excetra complete. (using NATURAL SAND).

Rate as per R. A. No.—	23

15893.40 One

Number

31786.80

Say Rs.

61,97,335.76

61,97,336.00

Name of Work : Construction of Godown Builidng For Chandaneshwar Agro Producer Company Ltd. In survey No 502 At:-Banawadi   Tal:-Koregaon   Dist:-   Satara. owner Name- t•tr. chandra Kisan Nalage.

M E A R S U R E M E N T

##### ITEM NO : 1

No.	Length	Breadth

### Height	Quantity

Excavation for foundation in earth, soils, SM, etc. (By Manual Means)

| 1 Vegetable Market |  |  |  | LW | 2.0 | 14.000 | 0.750 | 1.200 | 25.200 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 2.0 | 8.500 | 0.750 | 1.200 | 15.300 |
| 3 | Bakery | Unit | LW | 2.0 |  | 21.000 | 0.750 | 1.200 | 37.800 |
|  |  |  |  | 2.0 |  | 8.500 | 0.750 | 1.200 | 15.300 |
| 5 | Katta |  |  | 4.0 |  | 3.000 | 1.500 | 0.600 | 10.800 |
| 6 | Steps |  |  | 2.0 |  | 2.400 | 1.200 | 0.600 | 3.456 |
| 7 | Toilet | Unit | LW | 6.0 |  | 1.960 | 0.450 | 0.900 | 4.763 |
| 8 | SW |  |  | 6.0 |  | 1.000 | 0.450 | 0.900 | 2.430 |
| 9 | Grains | Unit |  | 2.0 |  | 21.000 | 0.750 | 1.200 | 37.800 |
| 10 | SW |  |  | 2.0 |  | 8.500 | 0.750 | 1.200 | 15.300 |
|  |  |  |  |  |  |  |  |  | 168.150 Cubic Metre |

ITEM NO : 2

Excavation for foundation in HM.

| 1 | Vegi Market | LW | 2.0 | 14.000 | 0.750 | 0.300 | 6 . 3 0 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2.0 | 8.500 | 0.750 | 0.300 | 3 . 8 2 5 |
| 3 | Bakery LW |  | 2.0 | 21.000 | 0.750 | 0.300 | 9. 4 5 0 |
|  |  |  | 2.0 | 8.500 | 0.750 | 0.300 | 3 . 8 2 5 |
| 5 | Grains Unit |  | 2.0 | 21.000 | 0.750 | 0.300 | 9. 4 5 0 |
| 6 | SW |  | 2.0 | 8.500 | 0.750 | 0.300 | 3 . 8 2 5 |
|  |  |  |  |  |  |  | 36.680 Cubic Metre |

##### ITEM NO : 3

Soling using 80 mm trap metal.

| 1 Vegi Market |  |  | LW | 2.0 | 14.000 | 0.750 | 0.300 | 6.300 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2.0 | 8.500 | 0.750 | 0.300 | 3.825 |
| 3 | Bakery | LW |  | 2.0 | 21.000 | 0.750 | 0.300 | 9.450 |
|  |  |  |  | 2.0 | 8.500 | 0.750 | 0.300 | 3.825 |
| 5 | Katta |  |  | 4.0 | 3.000 | 1.500 | 0.300 | 5.400 |
| 6 | Steps |  |  | 2.0 | 2.100 | 1.200 | 0.300 |  |
| 7 | Toilet | Unit | LW | 6.0 | 1.960 | 0.450 | 0.300 | 1.588 |
| 8 | SW |  |  | 6.0 | 1.000 | 0.450 | 0.300 | 0.810 |
| 9 | Grains | Unit |  | 2.0 | 21.000 | 0.750 | 0.300 | 9.450 |

10 SW

ITEM NO : 4

No.

2.0

###### Length	Breadth

8.500	0.750

###### Height	Quantity

0.300	3.825

45.990

Cubic Metre

P L	C.C. M—10 for foundation \ bedding without SCADA.—	With fine aggregate (Natural Sand Crushed sand VSI Grade finely washed etc) (using NAT. SAND).

| 1 Vegi Market |  |  | LW | 2.0 | 14.000 | 0.750 | 0.150 | 3.150 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2.0 | 8.500 | 0.750 | 0.150 | 1.913 |
| 3 | Bakery | LW |  | 2.0 | 21.000 | 0.750 | 0.150 | 4.725 |
|  |  |  |  | 2.0 | 8.500 | 0.750 | 0.150 | 1.913 |
| 5 | Katta |  |  | 4.0 | 3.000 | 1.500 | 0.150 | 2.700 |
| 6 | Steps |  |  | 2.0 | 2.100 | 1.200 | 0.150 | 0.756 |
| 7 | Toilet | Unit | LW | 6.0 | 1.960 | 0.450 | 0.100 | 0.529 |
| 8 | SW |  |  | 6.0 | 1.000 | 0.450 | 0.100 | 0.270 |
| 9 | Grains | Unit | LW | 2.0 | 21.000 | 0.750 | 0.150 | 4.725 |
| 10 | SW |  |  | 2.0 | 8.500 | 0.750 | 0.150 | 1.913 |
|  |  |  |  |  |  |  |  | 22.590 Cubic Metre |

ITEM NO : 5

Filling in plinth \ floors with excavated materials.

1 Qnty as per Item 2, Sub—Est 1	1.0 *	36.680 *

ITEM NO : 6

Filling in plinth & floors with contractors material.

*	———	=	36.680

36.680

Cubic Metre

| 1 Vegitable Market | 1.0 |  | 12.800 | 8.800 | 0.450 | 50.688 |
| --- | --- | --- | --- | --- | --- | --- |
| 2 Bakery And Grains Unit | 2.0 |  | 20.400 | 8.800 | 0.450 | 161.568 |
| 3 Less qnty as per Item 5, Sub—Est 1 | —1.0 | * | 36.680 | Total * | * ——— | 212.260 = —36.680 |
|  |  |  |  |  |  | 175.580 |

ITEM NO : 7

U.C.R. masonry in C.M. 1:6 in foundations \ plinth. (using NAT. SAND).

Cubic Metre

| 1 | Toilet | Unit | LW | 6.0 | 1.960 | 0.450 | 0.900 | 4.763 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 3.0 | 8.500 | 0.600 | 0.600 | 9.180 |
| 3 | Bakery | LW |  | 2.0 | 21.000 | 0.600 | 0.600 | 15.120 |
|  |  |  |  | 2.0 | 8.500 | 0.600 | 0.600 | 6.120 |
| 5 | Toilet | Unit | LW | 4.0 | 1.960 | 0.450 | 0.600 | 2.117 |
| 6 | SW |  |  | 4.0 | 1.000 | 0.450 | 0.600 | 1.080 |
|  |  |  |  |  |  |  |  | 38.380 Cubic Metre |

| IInd class |  | B.B | masonry | in | C.M. | 1:6 | in | foundations | \ | plinth. | (using NAT. | SAND). |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Katta |  |  |  |  |  |  |  | 4.0 |  | 3.000 | 0.230 | 0.750 | 2.070 |
| SW |  |  |  |  |  |  |  | 8.0 |  | 1.500 | 0.230 | 0.750 | 2.070 |
| Steps | 1 |  |  |  |  |  |  | 2.0 |  | 2.100 | 0.900 | 0.300 | 1.134 |
| Steps | 2 |  |  |  |  |  |  | 2.0 |  | 2.100 | 0.600 | 0.200 | 0 . 5 0 4 |
| Steps | 3 |  |  |  |  |  |  | 2.0 |  | 2.100 | 0.300 | 0.200 | 0 . 2 5 2 |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 6.030 Cubic Metre |
| ITEM NO : IInd class | 9 | B.B. | masonry in C.M. l:6 superstructure. |  |  |  |  |  |  | (For Ground Floor) (using NAT. SAND). |  |  |  |
| Vegi Market |  |  | LW | 2.0 |  |  |  |  | 14.000 |  | 0.230 | 1.500 | 9.660 |
|  |  |  |  | 1.0 |  |  |  |  | 9.540 |  | 0.230 | 1.500 | 3.291 |
| Bakery Unit |  |  | LW | 2.0 |  |  |  |  | 20.770 |  | 0.230 | 1.500 |  |

| BS |  | 1.0 | 9.540 | 0.230 | 1.500 | 3.291 |
| --- | --- | --- | --- | --- | --- | --- |
| Partation Wall |  | 2.0 | 9.540 | 0.230 | 3.600 | 15.798 |
| 8 Toilet Unit LW |  | 6.0 | 1.960 | 0.230 | 2.400 | 6.492 |
| 9 SW |  | 6.0 | 1.000 | 0.230 | 2.400 |  |
|  |  |  |  | Total |  | 56.180 |
| 6 Deduction For Shutter |  | —4.0 | 3.600 | 0.230 | 1.500 | —4.968 |
| 7 Deduction For Door |  | —4.0 | 1.200 | 0.230 | 1.500 | —1.656 |
|  |  |  |  | Less |  | —6.620 |
|  |  |  |  |  |  | 49.550 Cubic Metre |
| ITEM NO : 10 P L C.C. blocks of M—15 grade | flooring | laid in | C.M. 1:3 . | (using NAT. | SAND). |  |
| Vegi Market |  | 1.0 | 13.540 | 9.540 | 0.100 | 12.917 |
| Bakery |  | 1.0 | 20.770 | 9.540 | 0.100 | 19.815 |
| Grains Unit |  | 1.0 | 20.770 | 9.540 | 0.100 | 19.815 |
|  |  |  |  |  |  | 52.550 Cubic Metre |

PDF corrugated G.I. sheets 0.63 mm th (24 BWG) for roofing without wind tiles. (For Ground Floor)

| 1 | Sides Of Gowdown |  | 2.0 | 35.000 | 3.900 |  | 273.000 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | FS and BS Of Godown |  | 2.0 | 10.000 | 3.900 |  | 78.000 |
| 3 | Roofing For Godown |  | 1.0 | 35.000 | 10.300 |  | 360.500 |
| 7 | Sides Of Grains Unit | LW | 2.0 | 21.000 |  | 4.800 | 201.600 |
| 8 | Sides Of Grains Unit | SW | 1.0 | 10.000 |  | 4.800 | 48.000 |
| 9 | Grains Unit Roofing |  | 1.0 | 21.000 | 10.000 |  | 210.000 |
|  |  |  |  |  | Total |  | 1171.100 |
| 4 | Deduction For Shutter |  | —3.0 | 3.600 | 1.200 |  | —12.960 |
| S | Deduction For Doors |  | —4.0 | 1.200 |  | 0.600 | —2.880 |
| 6 | Deduction For Windows |  | —10.0 | 1.200 |  | 1.200 | —14.400 |
|  |  |  |  |  | Less |  | —30.240 |
|  |  |  |  |  |  |  | 1140.860 |

ITEM NO : 12

Structural steel work in trusses. (For Ground Floor)

Square Metre

1 Vertical Posts	I 125x70

3 Vertical Posts	I 125x70

11*2*5.40*13.30*1000

2*3*5.40*13.30*1000

11*2*5.40*13.30*1000

1.580

0.431

1.580

| 4 Horizontal Sq 50x50x4 | 2*4*10.00*5.72*1000 | 0.458 |
| --- | --- | --- |
| 5 Roofing C 100x50 | 11*10.00*9.56*1000 | 1.052 |
| 6 Angle 50x50x5 | 7*35.00*3.80*1000 | 0.931 |
| 7 Vertical Posts I 125x70 | 6*2*5.40*13.30*1000 | 0.862 |
| 8 | 1*3*5.40*13.30*1000 | 0.215 |
| 9 Horizontal Sq 50x50x4 | 2*5*21.00*5.72*1000 | 1.201 |
| 10 Horizontal Sq 50x50x4 | 2*5*4*10.00*5.72*1000 | 2.288 |
| 11 Roofing C 100x50 | 6*10.00*9.56*1000 | 0.574 |
| 12 Angle 50x50x5 | 7*21.00*3.80*1000 | 0.559 |
|  |  | 11.731 Metric Ton |
| ITEM NO : 13 PDF rolling shutter. — without | mechanical gear (For Ground Floor) |  |
|  | 4.0 3.600 2.700 | 38.880 |
|  |  | 38.880 Square Metre |

Internal C.P. 20 mm th single coat in C.M. 1:5. (For Ground Floors (using NAT. SAND).

| 1 Vegi Market | 2.0 | 13.540 | 1.500 | 40.620 |
| --- | --- | --- | --- | --- |
|  | 1.0 | 9.540 | 1.500 | 14.310 |
|  | 2.0 | 20.770 | 1.500 | 62.310 |
|  | 1.0 | 9.540 | 1.500 | 14.310 |
| 5 Partation Wall | 4.0 | 9.540 | 3.600 | 137.376 |
| 6 Toilet Unit LW | 6.0 | 1.500 | 2.400 | 21.600 |
| 7 SW | 6.0 | 1.000 | 2.400 | 14.400 |
| 8 Grains Unit LW | 2.0 | 20.770 | 1.500 | 62.310 |
| 9 SW | 2.0 | 9.540 | 1.500 | 28.620 |

ITEM NO : 15

Sand faced plaster. (For Ground Floor) (using NAT. SAND).

395.860

Square Metre

| 1 | LW |  | 2.0 | 35.000 | 1.500 | 105.000 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2.0 | 10.000 | 1.500 | 30.000 |
| 3 | Toilet | Unit | 6.0 | 1.960 | 2.400 | 28.224 |
|  |  |  | 6.0 | 1.000 | 2.400 | 14.400 |
| 5 | Grains | Unit | 2.0 | 21.000 | 1.500 | 63.000 |
| 6 | SW |  | 1.0 | 10.000 | 1.500 | 15.000 |

255.620

Square Metre

ITEM NO : 16

PAL	vitrified	matt finish tiles 590 to 605 X 590 to 605 X 8 to 10 mm th.—	Skirting (For Ground Floor) (using NAT. SAND).

- Vegi Market

- Bakery Unit

- Grains Unit

1.0

1.0

1.0

13.540

20.770

20.770

9.540

9.540

9.540

129.172

198.146

198.146

ITEM NO : 17

PDF M.S. grill work for windows, ventilator etc. 20 Kg*Sqm. (For Ground Floor)

525.460

Square Metre

ITEM NO : 18

15.0	1.200

1.200	21.600

21.600

Square Metre

PDF	aluminium	sliding	window	of	3 tracks. With colour anodising with box. (For Ground Floor)

15.0	1.200	1.200	21.600

21.600

Square Metre

ITEM NO : 19

PDF	solid	core	flush	door	shutter in single leaf 32 mm th decorative type of exterior grade, face veneers 3 mm thick on both faces. (For Ground Floor)

5.0	1.200

ITEM NO : 20

PDF CR steel hollow door frame 105 X 60 mm. (For Ground Floor)

6.0 *	1.200 *	*

6.0 *	2.100 *	*

2.050	12.300

12.300

Square Metre

7.200

12.600

19.800

Running Metre

ITEM NO : 21

PHA washable oil bound distemper to old \ new surfaces 2 coats. (For Ground Floor)

1 Qnty as per Item 13,  Sub—Est 1	1.0 *	395.860 *	*	———	=	395.860

ITEM NO : 22

395.860

Square Metre

PHA 2 coats of exterior weather shield paint to plastered surfaces. (For Ground Floor)

1 Qnty as per Item 14,  Sub—Est 1	1.0 *	255.620 *	*	———	=	255.620

ITEM NO : 23

255.620

Square Metre

P C M—20 R.C.C. slabs \ landings with batch mixer without SCADA.— With fine aggregate (Natural Sand Crushed sand VSI Grade finely washed etc) (For Ground Floor) (using NAT. SAND).

1 WC Unit	3.0	1.960	1.000	0.100	0.588

ITEM NO : 24

0.590

Cubic Metre

PDF	orissa	type	colour	glazed	earthenware	625	X	450 mm W.C. pan with 10 lit P.V.C. flushing cistern. (For Ground Floor)

1 WC Pan	3.0	**

ITEM NO : 25

PDF white glazed earthenware W.H.B. 55 X 40 cm. (For Ground Floor)

3.0

ITEM NO : 26

P*L*F P.V.C. pipe 15 mm dia including necessary excavation, trench filling.

3.000

3.000

Number

3.000

3.000

Number

1.0

45.000

45.000

ITEM NO : 27

P*L*F P.V.C. pipe 110 mm dia including necessary excavation, trench filling.

1.0 *	23.000 *	*

45.000

Running Metre

23.000

23.000

Running Metre

ITEM NO : 28

##### P F	H.D.P.	container	1 piece moulded water tank—	Capacity 200 to 1000 lit. (For Ground Floor)

ITEM NO : 29

##### PDF hume pipe septic tank 900 mm dia.

3.0	500.000

1.0 *	*	*

———	= 1500.000

1500.000

##### Litre

1.000

1.000

Number

ITEM NO : 30

Brick masonry inspection chamber 90 X 45 X 90 cm with C.I. lid 75 kg. (using NAT. SAND).

2.0	**	2.000

2.000

Number

### Agro Producer Company Ltd. In survey No 502 At:-Banawadi   Tal:-Koregaon   Dist:-   Satara. owner Name- t•tr. chandra Kisan Nalage.

L E A D	S T A T E M E N T

Sr	Material

No

Additial Lead Charges

Lead in	Lead	Initial	Net Lead

K. M.	Charge	Lead	Charge

Source

| Soling |  | stone |  | 22.000 |  |  |  | 549.46 | 0.00 | 549.46 | Limbkhind |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Crushed |  | Sand |  | 22.000 |  |  |  | 449.13 | 0.00 | 449.13 | Limbkhind |
| Cement |  |  |  | 5.000 |  |  |  | 148.81 | 0.00 | 148.81 | Local |
| Sand |  |  |  | 15.000 |  |  |  | 355.50 | 0.00 | Z55.50 |  |
| Metal below |  |  |  | 40 | mm | 22.000 |  | 449.13 | 0.00 | 449.13 | Limbkhind |
| 6 Hard murum |  |  |  |  |  | 5.000 |  | 220.06 | 0.00 | 220.06 | Local |
| 7 Bricks |  |  |  |  |  | 55.000 |  | 1.92601 | 0.00000 | 1.92601 | Umbraj |
| 8 | Structural |  | Steel |  |  | 20.000 |  | 309.96 | 0.00 | 309.96 | Koregaon |
| 9 Screened Sand |  |  |  |  |  |  | 22.000 | 449.13 | 0.00 | 449.13 | Limbkhind |
| 10 Flooring Tiles |  |  |  |  |  |  | 20.000 | 31.00 | 0.00 | 31.00 | Koregaon |
| 11 R.C.C. Pipes 900 |  |  |  |  |  | mm. dia. | 25.000 | 125.03 | 0.00 | 125.0300 | Satara |

Agro Producer Company Ltd. In survey No 502 At:-Banawadi  Tal:-Koregaon  Dist:-  Satara. owner Name- t•tr. chandra Kisan Nalage.

R A T E	A N A L Y S I S

Providing soling using 80 mm size trap metal in 15 cm layer. complete.

Completed  Item Rate as per SSR 2021—22 UP	21.40*118

Deduct 2.0% for Labour aminities & Cess	(—2.0	100 * 1322.00)

Add 10.00% of Rs. 1295.56 For Hilly*Inacce. Area On DSR Rate

1322.00

—26.44

1295.56

129.56

Total Additional Charges

Add for Soling stone for 22.000 KM

549.46	0.00) =	549.46	1.000000

Add for Crushed Sand for 22.000 KM

449.13	0.00)	449.1Z	0.150000

Add Back 2.0% for Labour aminities \ Cess

549.46

67.37

616.83

(2.0	100 * 1322.00)

616.83

2041.95

26.44

2068.39

Total Rate Rs.	2,068.40

Per Cubic Metre

Providing & laying in situ, C.C. M—10 of trap* granite* quartzite* gneiss metal for foundation \ bedding. complete, with fully automatic micro processor based PLC without SCADA enabled reversible Drum Type mixer.—  With fine aggregate (Natural Sand  Crushed sand VSI Grade finely washed etc) (using NATURAL SAND).

Completed Item Rate as per SSR 2021—22 UP 24.01*131	5538.00

Deduct Rs. 120.00 for SCADA for concrete items to be used without SCADA

(—120.00)	—120.00

Deduct 2.0% for Labour aminities & Cess	(—2.0	100 * (5538.00	120.00)) =	—108.36

Add 10.00% of Rs. 5309.64 For Hilly*Inacce. Area On DSR Rate

5309.64

530.96

Total Additional Charges

Add for Cement for 5.000 KM 148.81	0.00)

Add for Sand for 15.000 KM

148.81 *	0.220000

32.74

5840.60

622.01

355.50	0.00)	355.50	0.470000

Add for Metal below 40 mm for 22.000 KM

449.13	0.00)	449.13	0.940000

167.09

422.18

622.01

Add Back 2.0% for Labour aminities & Cess	(2.0	100 * (5538.00	120.00))

6462.61

108.36

6570.97

Total Rate Rs.	6,570.95

Per Cubic Metre

Filling in plinth & floors with contractors material complete.

| Completed Item Rate as per SSR 2021—22 UP Deduct 2.0% for Labour aminities & Cess | 21.37*117 | (—2.0 | 100 | * 550.00) | 550.00 —11.00 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 539.00 |
| Add 10.00% of Rs. 539.00 For Hilly*Inacce. | Area On DSR | Rate |  |  | 53.90 |
|  |  |  |  |  | 592.90 |
| Total Additional Charges |  |  |  |  | 220.06 |

Add for Hard murum for 5.000 KM

220.06	0.00)	220.06 *	1.000000

Add Back 2.0% for Labour aminities \ Cess

220.06

220.06

(2.0	100 * 550.00)

812.96

11.00

8 P 8 . 9 6

Total   Rake Rs .	823. 95

P er	Cubi c  Year e

R.A. NO : 9

Providing IInd class B.B. masonry with conventional I.S. type bricks in C.M. 1:6 in foundations \ plinth of inner walls in plinth of external walls including bathing out water, striking joints, on unexposed faces, racking out joints on exposed faces & watering complete. (using NATURAL SAND).

Completed Item Rate as per SSR 2021—22 UP	27.01*144 Deduct 2.0% for Labour aminities & Cess

7343.00

(—2.0	100 * 734Z.00) =	—146.86

7196.14

|  |  |  |  |  |  | 7915.75 |
| --- | --- | --- | --- | --- | --- | --- |
| Total | Additional Charges |  |  |  |  | 1040.37 |
| Add | for Cement for 5.000 KM |  |  |  |  |  |
|  | 148.81 0.00) |  | 148.81 | 0.079000 | 11.76 |  |
| Add | for Sand for 15.000 KM |  |  |  |  |  |
|  | 355.50 0.00) |  | 355.50 | 0.320000 | 113.76 |  |
| Add | for Bricks for 55.000 KM |  |  |  |  |  |
| 1.92601 |  | 0.00000) | 1.92601 | *475.000000 | 914.85 |  |

Add 10.00% of Rs. 7196.14 For Hilly*Inacce. Area On DSR Rate	719.61

Add Back 2.0% for Labour aminities \ Cess

1040.37

(2.0	100 * 734Z.00)

8956.12

146.86

9102.98

Total Rate Rs.	9,103.00

Per Cubic Metre

Providing IInd class B.B. masonry with conventional I.S. type bricks in C.M. 1:6 superstructure including striking joints, racking out joints, watering \ scaffolding etc. complete. (For Ground Floor) (using NATURAL SAND).

Completed Item Rate as per SSR 2021—22 UP	27.05*144 Deduct 2.0% for Labour aminities & Cess

7570.00

(—2.0	100 * 7570.00) =	—151.40

7418.60

Add 10.00% of Rs. 7418.60 For Hilly*Inacce. Area On DSR Rate

Total Additional Charges

Add for Cement for 5.000 KM

741.86

8160.46

1040.37

148.81	0.00)

Add for Sand for 15.000 KM 355.50	0.00)

Add for Bricks for 55.000 KM 1.92601	0.00000)

148.81	0.079000

355.50	0.320000

1.92601 *475.000000

11.76

113.76

914.85

1040.37

9200.83

Add Back 2.0% for Labour aminities \ Cess

(2.0	100 * 7570.00)

151.40

9352.23

Total Rate Rs.	9,352.25

Per Cubic Metre

Providing	\	laying	C.C. blocks of M—15 grade flooring laid in C.M. 1:3 as per drawing technical specifications. (using NATURAL SAND).

Completed Item Rate as per SSR 2021—22 UP	14.77*76 Deduct 2.0% for Labour aminities & Cess

7144.00

(—2.0	100 * 7144.00) =	—142.88

Add 10.00% of Rs. 7001.12 For Hilly*Inacce. Area On DSR Rate

7001.12

700.11

Total Additional Charges

Add for Cement for 5.000 KM 148.81	0.00)

Add for Sand for 15.000 KM

148.81 *	0.330000

49.11

7701.23

590.85

355.50	0.00)	355.50	0.450000

Add for Metal below 40 mm for 22.000 KM

449.13	0.00)	449.13	0.850000

159.98

381.76

Add Back 2.0% for Labour aminities \ Cess

590.85

(2.0	100 * 7144.00)

8292.08

142.88

8 48 4 . 9 6

Total   Rake  Rs .	8, d3d . 95

P er	Cubi c  Year e

Providing	structural	steel work in trusses, other similar trussed purlins, etc. \ 1 coat of anticorrosive paint a 2 coats of oil painting complete. (For Ground Floor)

Completed Item Rate as per SSR 2021—22 UP	23.04*129 Deduct 2.0% for Labour aminities & Cess

105909.00

(—2.0	100 * 105909.00) =	—2118.18

103790.82

Add 10.00% of Bs. 103790.82 For Hilly*Inacce. Area On DSR Rate

Total Additional Charges

Add for Structural Steel for 20.000 KM

(	309.96	0.00)	309.96	1.000000

309.96

10379.08

11 41 6 9.  9 0

8 0 9.  9 6

Add Back 2.0% for Labour aminities \ Cess

309.96

(2.0	100 * 105909.00)

114479.86

2118.18

116598.04

Total Rate Rs.	1,16,598.05

Per Metric Ton

Providing internal C.P. 20 mm thick in single coat in C.M. 1:5 without neeru finish, to concrete, brick surface, in all positions. complete. (For Ground Floor) (using NATURAL SAND).

Completed  Item Rate as per SSR 2021—22 UP	32.06*153

Deduct 2.0% for Labour aminities & Cess	(—2.0	100 * 359.00)

Add 10.00% of Rs. 351.82 For Hilly*Inacce. Area On DSR Rate

Total Additional Charges

Add for Cement for 5.000 KM

359.00

—7.18

351.82

35.18

387.00

12.27

# i48 8i	o oo)	i48 8i	o oo7ooo

Add for Screened Sand for 22.000 KM

449.13	0.00)	449.13	0.025000

1.04

11.23

Add Back 2.0% for Labour aminities \ Cess

12.27

(2.0	100 * 359.00)

399.27

7.18

406.45

Total Rate Rs.	406.45

Per Square Metre

Providing sand faced plaster externally in C.M. using approved screened sand in all positions including base coat of 15 mm thick in C.M. l:4 using W.P. compound at 1 Kg per cement bag curing same for not less than 2 days & keeping surface of base coat rough to receive sand faced treatment of 6 to 8 mm thick in C.M. 1:4 finishing surface by taking out grains \ curing for fourteen days, scaffolding etc. complete. (For Ground Floor) (using NATURAL SAND).

Completed  Item Rate as per SSR 2021—22 UP	32.11 153

Deduct 2.0% for Labour aminities & Cess	(—2.0	100 * 599.00)

Add 10.00% of Rs. 587.02 For Hilly*Inacce. Area On DSR Rate

Total Additional Charges

Add for Cement for 5.000 KM

599.00

—11.98

587.02

58.70

645.72

16.68

# i48 8i	o oo)	i48 8i	o oicsoo

Add for Screened Sand for 22.000 KM

449.13	0.00)	449.13	0.033000

1.86

14.82

Add Back 2.0% for Labour aminities \ Cess

16.68

(2.0	100 * 599.00)

662.40

11.98

674.38

Total Rate Rs.	674.40

Per Square Metre

Providing & laying vitrified matt finish tiles having size 590 to 605 X to 605 mm of 8 to

10 mm thick a confirming to IS 15622—2006 (Group BIa) of approved make, shade \ pattern for flooring in required position laid on bed of 1:4 C.M. including neat cement float, filling joints, curing & cleaning etc. complete. — Skirting (For Ground Floor) (using NATURAL SAND).

Completed  Item Rate as per SSR 2021—22 UP	33.43*159

Deduct 2.0% for Labour aminities & Cess	(—2.0	100 * 1982.00)

1982.00

—39.64

Add 10.00% of Bs. 1942.36 For Hilly*Inacce. Area On DSR Rate

Total Additional Charges

Add for Cement for 5.000 KM

(	148.81	0.00)	148.81	0.010000

Add for Sand for 15.000 KM

355.50	0.00)	355.50 *	0.017000

Add for Flooring Tiles for 20.000 KM

31.00	0.00) =	31.00 *	1.000000

1.49

6.04

31.00

38.53

2136.60

38.53

Add Back 2.0% for Labour aminities \ Cess

(2.0	100 * 1982.00)

2175.13

39.64

2214.77

Total Rate Rs.	2,214.75

Per Square Metre

R.A. NO : 16

Providing \ fixing M.S. grill work for windows, ventilator etc. 20 Kg*Sqm as per drawings including fixtures, necessary welding \ painting with 1 coats of anticorrosive paint \ two coats of oil painting etc. complete. (For Ground Floor)

Completed  Item Rate as per SSR 2021—22 UP	40.01*192

Deduct 2.0% for Labour aminities & Cess	(—2.0	100 * 1845.00)

Add 10.00% of Rs. 1808.10 For Hilly*Inacce. Area On DSR Rate

Total Additional Charges

Add for Structural Steel for 20.000 hM

1845.00

—36.90

1808.10

180.81

1988.91

6.73

309.96	0.00)	309.96	0.021700

6.73

6.73

Add Back 2.0% for Labour aminities \ Cess

(2.0	100 * 1845.00)

1995.64

36.90

R.A. NO : 20

2032.54

Total Rate Rs.	2,032.55

Per Square Metre

Providing U.C.R. masonry of trap granite quartzite gneiss stones in C.M. 1:6 in foundations \ plinth of inner walls in plinth of external walls including bathing out water, striking joints on unexposed faces & watering complete. (using NATURAL SAND).

| Completed Item Rate as per SSR 2021—22 UP 28.01*146 Deduct 2.0% for Labour aminities & Cess (—2.0 | 100 | * 4757.00) | 4757.00 —95.14 |
| --- | --- | --- | --- |
|  |  |  | 4661.86 |
| Add 10.00% of Rs. 4661.86 For Hilly*Inacce. Area On DSR Rate |  |  | 466.19 |

Total Additional Charges

Add for Cement for 5.000 KM 148.81	0.00)

Add for Sand for 15.000 KM

148.81 *	0.085000

12.65

5128.05

796.43

355.50	0.00)	355.50	0.350000

Add for Soling stone for 22.000 KM

549.46	0.00) =	549.46	1.200000

659.35

Add Back 2.0% for Labour aminities \ Cess

796.43

(2.0	100 * 4757.00)

5924.48

95.14

6019.62

Total Rate Rs.	6,019.60

Per Cubic Metre

Providing \ laying cast in situ Ready Mix C.C. M—20 of trap* granite quartzite* gneiss metal for R.C.C. slabs & landings as per detailed designs & drawings including steel centering, formwork, cover blocks, compaction, finishing formed surfaces with C.M. 1:3 of sufficient min. thickness to give smooth & even surface or roughening surface if special finish is to be provided & curing, etc. complete. (Excluding reinforcement & structural steel). with fully automatic micro processor based PLC without SCADA enabled reversible Drum Type mixer* concrete Batch mix plant (Pan mixer) etc. complete. With fine aggregate (Natural Sand Crushed sand VSI  Grade  finely washed etc) (For Ground Floor) (using NATURAL SAND).

Completed  Item Rate as per SSR 2021—22 UP  25.70*135	13943.00

Deduct Rs. 120.00 for SCADA for concrete items to be used without SCADA

(—120.00)	—120.00

Deduct 2.0% for Labour aminities & Cess (—2.0	100 * (13943.00	120.00)) =	—276.46

13546.54

Add 10.00% of Bs. 13546.54 For Hilly*Inacce. Area On DSR Rate

Total Additional Charges

Add for Cement for 5.000 KM

(	148.81	0.00)	148.81	0.350000

Add for Sand for 15.000 KM

355.50	0.00)	355.50 *	0.425000

Add for Metal below 40 mm for 22.000 KM

449.13	0.00)	449.1Z	0.850000

52.08

151.09

381.76

584.93

1354.65

1 49 0 I . 1 9

E 8 4 . 98

Add Back 2.0% for Labour aminities & Cess	(2.0	100 * (13943.00	120.00))

15486.12

276.46

15762.58

Total Rate Rs.	15,762.60

Per Cubic Metre

Providing	&		fixing	hume	pipe	septic	tank	900 mm dia with vent pipe & cap, including excavation		laying.

Completed Item Rate as per SSR 2021—22 UP	42.18*201 Deduct 2.0% for Labour aminities & Cess

15690.00

(—2.0	100 * 15690.00) =	—313.80

15376.20

Add 10.00% of Rs. 15376.20 For Hilly*Inacce. Area On DSR Rate

Total Additional Charges

Add for R.C.C. Pipes 900 mm. dia. for 25.000 KM

(	125.03	0.00) =	125.0Z *	2.500000

312.58

312.58

1537.62

16913.82

312.58

Add Back 2.0% for Labour aminities \ Cess

(2.0	100 * 15690.00)

Total Rate Rs.

17226.40

313.80

17540.20

###### 17,540.20

Per Number

Providing	\ constructing brick masonry inspection chamber 90 X 45 X 60 cm including 1:4:8

C.C.	foundation, 1:2:4 C.C. channels, half round G.S.W. pipes, Brick masonary, plastering from inside \ C.I. lid cover 75 kg with frame fixed in C.C.. (using NATURAL SAND).

Completed Item Rate as per SSR 2021—22 UP	42.11*200 Deduct 2.0% for Labour aminities & Cess

13229.00

(—2.0	100 * 13229.00) =	—264.58

12964.42

Add 10.00% of Rs. 12964.42 For Hilly*Inacce. Area On DSR Rate	1296.44

|  |  |  |  |  |  | 14260.86 |
| --- | --- | --- | --- | --- | --- | --- |
| Total | Additional Charges |  |  |  |  | 1367.98 |
| Add | for Cement for 5.000 KM |  |  |  |  |  |
| ( | 148.81 0.00) |  | 148.81 * | 0.176500 | 26.26 |  |
| Add | for Sand for 15.000 KM |  |  |  |  |  |
|  | 355.50 0.00) |  | 355.50 * | 0.540000 | 191.97 |  |
| Add | for Metal below 40 mm for |  | 22.000 KM |  |  |  |
|  | 449.13 0.00) |  | 449.13 * | 0.300000 | 134.74 |  |
| Add | for Bricks for 55.000 KM |  |  |  |  |  |
| ( | 1.92601 | 0.00000) | 1.92601 *527.000000 |  | 1015.01 |  |

1367.98

Add Back 2.0% for Labour aminities \ Cess

(2.0	100 * 13229.00)

Total Rate Rs.

15628.84

264.58

15893.42

###### 15,893.40

Per Number